Formwork Solutions at the Meldahl Hydroelectric Project

Featuring approximately 40,000 square feet of Doka’s pre-assembled Top 50 gang formwork, the Meldahl Project is designed to provide a new source of clean, zero emission electric generation for the national grid.

Located on the Ohio River, approximately 40 miles east of Cincinnati at Foster, Kentucky, the Meldahl Project includes a 105 megawatt "run-of-the-river" hydroelectric plant. The challenge was to deliver this monumental project efficiently on an aggressive three-month schedule. As such, Doka was hired by Baker Concrete Construction to meet the challenge.

Design

The project design called for three, horizontal draft tubes with a conical shape starting with an approximate diameter of 36’-6” and transitioning into a rectangle at 59’ x 54’, with a total length of 91’-9”. Doka combined the efforts of their Chicago and Baltimore branches to meet these close tolerances and ensure the project was completed on time. Construction of the draft tubes began using Top 50 Formwork, a loose form system constructed from standard parts that can be assembled in any configuration for a wide range of applications. To allow for concrete to be placed faster, (522) pre-assembled, conical Top 50 gangs were delivered onsite and ready for immediate installation.

Pre-assembly

All Top 50 gang formwork that form the interior conical shape of the draft tubes were built and preassembled at Doka’s facility in Channahon, IL.

Doka’s experienced staff and state of the art carpentry shop worked day and night to construct all 522 individual gangs. Since each gang had a specific shape and location within the tube, all were individually numbered and shipped according to its pour ID number to provide maximum efficiency at the jobsite.

A grill of walers and beams was first dimensioned and assembled for each gang, then, the CNC machine shaping timbers were constructed to precise dimensions and attached to each grill along with a plywood form face. Next, the gang was labeled and the plywood face was covered with plastic for storage in Doka’s yard to provide quality and just-in-time delivery at the jobsite.

Load-bearing Tower Staxo 100

Doka’s Staxo 100 shoring was installed prior to the roof formwork assembly. This high performance, load-bearing tower is quick and easy to assemble and can be completed in a variety of ways to fit a wide range of field conditions. In addition, the flexibility of the system allows for maximum optimization of equipment and a reduction in necessary material. One million pounds of Staxo 100 Shoring was delivered and allowed for onsite pre-assembly for each level and segment.

"Staxo shoring is the safest system that I have ever used. The built in ladder, tie-offs and horizontal cross bracing make it a good system," said Kirt Inscho, superintendent, Baker Concrete Construction.

Staying on Schedule

To meet the demanding construction schedule, nearly 80 truckloads of Top 50 and Staxo 100 were delivered in total. To ensure safety on the jobsite, Doka’s field service was provided to educate and assist the crew on proper assembly of both Top 50 and Staxo 100.

"Doka's on-site field support was important to the success of this project,” said Inscho.

According to Michael Schaeffer, National Sales Manager, Doka’s experience on the Cannelton Hydroelectric Project assisted in the success of this project.

The Meldahl Project represents a significant economic development investment for the local community. More than 400 construction jobs were created for a variety of trade workers. The project is slated for commercial operation in 2014.

About Doka

With innovations and more than 50 years’ experience in formwork engineering, Doka is one of the world’s leading single-source suppliers of complete formwork solutions. Doka has a comprehensive range of products and services with which it can offer economical formwork solutions in practically all areas of casting concrete construction, ranging  from multifamily residential buildings and high-rises to complex infrastructure projects such as bridges, water treatment plants, and power stations. For more information, please visit www.doka.com.

# # #

Facts:

Location: Foster, Kentucky

Products: Three draft tubes, 40,000 square feet of Doka Top 50 Formwork, One million pounds of Staxo Shoring.

General Contractor: Alberici / Baker JV

Concrete Contractor: Baker Concrete Construction of Monroe, Ohio

Overall project cost: $504 Million

Start date and scheduled end of date: May 2012-January 2013

Meldahl Hydroelectric Project 02.JPG - 522 pre-assembled, conicalTop 50 gangs were delivered onsite.

Meldahl Hydroelectric Project 01.JPG

Meldahl Hydroelectric Project 05.JPG

Meldahl Hydroelectric Project 04.JPG - Featuring over 40,000 sq.ft. of Doka’s pre-assembled Top 50 gang formwork, the Meldahl Project is designed to provide a new source of clean, zero emission electric generation for the national grid.

Meldahl Hydroelectric Project 03.JPG - Over 1 million pounds of Staxo was delivered and allowed for on-site pre-assembly for each level and segment